# SYREN NETWORKS

## Small Office / Home Office (SOHO) Network

### Design & Implementation Report

## 1. Project Overview

XYZ Company is a fast-growing food business that is based in Eastern Australia, it serves over 2 million customers worldwide. The company is opening a new branch near the village of Bonalbo, and need a new brand-new network  built from scratch – completely separate from the main headquarters network.

  - Network Requirements

- Use one CISCO router and one Cisco switch

- Have 3 separate departments, each on its own VLAN (Virtual Local Area Network)

- Each department must have a wireless access point (Wi-Fi) for devices

- All devices must get their IP addresses automatically (using DHCP)

- All departments must be able to talk to each other across VLANs

- We assume the ISP gave us the IP address 192.168.1.0

Underneath is the network problem:

1.2 The 3 Departments

| Department | VLAN Number | Wi-Fi Password | Security |
| --- | --- | --- | --- |
| ADMIN/IT | VLAN 10 | Admin@SyrenNetwork123 | WPA2-PSK |
| FINANCE/HR | VLAN 20 | Finance@123 | WPA2-PSK |
| CS/RECEPTION | VLAN 30 | Customer@123 | WPA2-PSK |

## 2. IP Address Planning (Subnetting)

The ISP gave us the base network address 192.168.1.0. Since we have 3 departments, we needed to split (subnet) into 3 smaller networks – one for each department.

### 2.1 How We Calculate the Subnets

We use the VLSM (Variable Length Subnet Masking). Here is the step-by-step breakdown:

We need 3 subnets (one per department)

To get 3 more subnets we borrow 2 bits from the host portion (2² = 4 possible subnets, we use 4)

Original subnet mask (Class C): 255.255.255.0   binary: 11111111.11111111.11111111.00000000

After borrowing 2 bits the new binary form subnet mask becomes 11111111.11111111.11111111.11000000

In order for us to get the last octet we need to do the following

We raise to the power of and multiply by the bit

In order for us to get the last octet of the subnet mask we need to multiply with numbers from the right.

From 0 to 7

^ raised to

2 ^ 0 * 0 = 0

2 ^ 1 * 0 = 0

2 ^ 2 * 0 = 0

2 ^ 3 * 0 =0

2 ^ 4 * 0 = 0

2 ^ 5 * 0 = 0

2 ^ 6 * 1 = 64

2 ^ 7 * 1 = 128

64 + 128 = 192

The new subnet mask becomes 255.255.255.192 Here is the list of block sizes

Block size tells us how to move from one subnet mask to another

In this case the block size  64

### 2.2 Subnet Table

| VLAN | Department | Network ID | Host Range | Broadcast | Hosts |
| --- | --- | --- | --- | --- | --- |
| VLAN 10 | Admin/IT | 192.168.1.0 | 192.168.1.1-192.168.1.62 | 192.168.1.63 | 62 |
| VLAN 20 | Finance/HR | 192.168.1.64 | 192.168.1.65-192.168.1.126 | 192.168.1.127 | 62 |
| VLAN 30 | CS/Reception | 192.68.1.128 | 192.168.1.129-192.168.1.190 | 192.168.1.191 | 62 |

In order for us to get the Network ID we multiply network ID one by the block size.

The broadcast ID is the last IP address before we move to the next subnet. (the network ID in this case)

The range of valid host lies between the network ID and the Broadcast ID

### 3. VLAN Configuration (Switch Setup)

A VLAN (Virtual Local Area Network) is like drawing invisible walls between departments on the same physical switch. Devices in VLAN 10 cannot accidentally talk to devices in VLAN 20 or 30 without going through the router first.

### 3.1 Port Assignment

The port assignment was done directly on the switch using the CLI. Here is exactly what was typed:

! --- Enable privileged mode --- Switch> en Switch# conf t ! --- VLAN 10: Admin/IT (ports Fa0/2, Fa0/3, Fa0/4) --- Switch(config)# int range fa0/2-4 Switch(config-if-range)# switchport mode access Switch(config-if-range)# switchport access vlan 10 ! Output: % Access VLAN does not exist. Creating vlan 10 ! --- VLAN 20: Finance/HR (ports Fa0/5, Fa0/6, Fa0/7) --- Switch(config-if-range)# int range fa0/5-7 Switch(config-if-range)# switchport mode access Switch(config-if-range)# switchport access vlan 20 ! Output: % Access VLAN does not exist. Creating vlan 20 ! --- VLAN 30: CS/Reception (ports Fa0/8, Fa0/9, Fa0/10) --- Switch(config-if-range)# int range fa0/8-10 Switch(config-if-range)# switchport mode access Switch(config-if-range)# switchport access vlan 30 ! Output: % Access VLAN does not exist. Creating vlan 30 ! --- Save the configuration --- Switch(config-if-range)# do wr ! Output: Building configuration... [OK]

### 3.2 Switch Commands Used

Switch> en Switch# conf t ! --- Assign ports to VLAN 10 (Admin/IT) --- Switch(config)# int range fa0/2-4 Switch(config-if-range)# switchport mode access Switch(config-if-range)# switchport access vlan 10 ! --- Assign ports to VLAN 20 (Finance/HR) --- Switch(config-if-range)# int range fa0/5-7 Switch(config-if-range)# switchport mode access Switch(config-if-range)# switchport access vlan 20 ! --- Assign ports to VLAN 30 (CS/Reception) --- Switch(config-if-range)# int range fa0/8-10 Switch(config-if-range)# switchport mode access Switch(config-if-range)# switchport access vlan 30 Switch(config-if-range)# do wr

### 3.3 Configure the Trunk Port

The port connecting the switch to the router must be set to TRUNK mode. This allows traffic from ALL VLAN’s to travel through a single cable to the router.

Switch(config)# int fa0/1 Switch(config-if)# switchport mode trunk Switch(config-if)# do wr

### 4. Router Configuration (Inter-VLAN Routing)

Even though the departments are separated by VLANS, they still need to communicate with each other. This Is called Inter-VLAN Routing. We use the router to act as a traffic controller between VLANs.

We use a technique called Router-on-a-Stick – one physical router port (Gig0/0) is split into 3 virtual sub-interfaces, each one assigned to a VLAN.

### 4.1 Enable the Physical Interface

Router> en Router# conf t Router(config)# int gig0/0 Router(config-if)# no sh Router(config-if)# do wr

### 4.2 Create Sub-Interfaces each VLAN

! --- Sub-interface for VLAN 10 (Admin/IT) --- Router(config)# int gig0/0.10 Router(config-subif)# encapsulation dot1Q 10 Router(config-subif)# ip address 192.168.1.1 255.255.255.192 Router(config-subif)# ex ! --- Sub-interface for VLAN 20 (Finance/HR) --- Router(config)# int gig0/0.20 Router(config-subif)# encapsulation dot1Q 20 Router(config-subif)# ip address 192.168.1.65 255.255.255.192 Router(config-subif)# do wr Router(config-subif)# ex ! --- Sub-interface for VLAN 30 (CS/Reception) --- Router(config)# int gig0/0.30 Router(config-subif)# encapsulation dot1Q 30 Router(config-subif)# ip address 192.168.1.129 255.255.255.192 Router(config-subif)# do wr

Each sub-interface gets the first usable IP in its subnet. This becomes the default gateway for all devices in that department

### 5. DHCP Configuration (Automatic IP Assignment)

Instead of manually typing an P address on every device, DHCP (Dynamic Host Configuration Protocol) does it automatically. When a device connects to the network, the router hands it an IP address from the correct pool.

### 5.1 DHCP Commands on the Router

Router(config)# service dhcp ! --- DHCP Pool for Admin/IT (VLAN 10) --- Router(config)# ip dhcp pool Admin-Pool Router(dhcp-config)# network 192.168.1.0 255.255.255.192 Router(dhcp-config)# default-router 192.168.1.1 Router(dhcp-config)# dns-server 192.168.1.1 Router(dhcp-config)# domain-name Admin.com Router(dhcp-config)# exit ! --- DHCP Pool for Finance/HR (VLAN 20) --- Router(config)# ip dhcp pool Finance-Pool Router(dhcp-config)# network 192.168.1.64 255.255.255.192 Router(dhcp-config)# default-router 192.168.1.65 Router(dhcp-config)# dns-server 192.168.1.65 Router(dhcp-config)# domain-name Finance.com Router(dhcp-config)# exit ! --- DHCP Pool for CS/Reception (VLAN 30) --- Router(config)# ip dhcp pool CS-Pool Router(dhcp-config)# network 192.168.1.128 255.255.255.192 Router(dhcp-config)# default-router 192.168.1.129 Router(dhcp-config)# dns-server 192.168.1.129 Router(dhcp-config)# domain-name CS.com Router(dhcp-config)# exit Router(config)# do wr

6. Wireless Access Point Configuration

Each department has its own Wireless Access Point (WAP). This lets phones, laptops, tables, and other wireless devices connect to their VLAN without needing a cable.

6.1 How to Set Up Each Access Point

Open the Access Point in Cisco Packet Tracer

Go to Port 1 settings

Set the Security Mode to WPA2-PSK

Enter the password for that department (see table in Section 1.2)
WPA2-PSK is the gold standard for Wi-Fi security. It encrypts all wireless traffic, meaning only people with the correct password can join the network.

### 7. Testing & Verification

After everything was configured, we tested whether devices could communicate across VLANs using the ping command.

### 7.1 How to Test

On any PC, go to Desktop  IP Configuration and set it to DHCP

The PC should automatically receive the IP address from the router

Open the Command Prompt and ping a device in a different VLAN

A successful reply means Inter-V LAN Routing Is working correctly

! Example: From Admin PC (VLAN 10) ping Finance PC (VLAN 20) C:\> ping 192.168.1.65 ! Expected result: Reply from 192.168.1.65: bytes=32 time<1ms TTL=255 Reply from 192.168.1.65: bytes=32 time<1ms TTL=255 Reply from 192.168.1.65: bytes=32 time<1ms TTL=255

### 8. Summary

Here is a quick recap of everything that was built and configured:

Subnetting: Divided 192.168.1.0 into 3 subnets using a slash notation of 26 (255.255.255.192)

Switch VLANS: Created VLAN 10,20, and 30 and assigned the correct ports

Trunk Port: Set Fa0/1 as a trunk so all VLANs can reach the router

Router Sub-Interfaces: Created Gig0/0. 10, .20, and .30 for Inter-VLAN Routing

DHCP: Configured 3address pools so all devices get automatic Ips

Wireless: Configured WPA2-PSK on 3 access points – one per department

Testing: Verified cross-VLAN communication with ping

The network is fully operational, secure and scalable. Each department is isolated on its own VLAN for security, while still being able to communicate through the router.